2026 Pure Stock RULES

"Rusty Bolt"

​

CAR REQUIREMENTS

- Any V8 cylinder car/truck providing it meets all safety requirements listed in the general rules.

- No 4 wheel or all wheel drives.

- No Turbos, Superchargers, or Nitrous are allowed.

- Gas only NO MENTHANOL OR ALCOHOL!

- Car MUST weigh 3200lbs with driver after race.

BODY & FRAME

- May run stock or aftermarket bodies. (No late model noses)

- May run a maximum of a 5in spoiler.

- The front firewall must remain stock, and in the stock location.

- Must have stock floor pans from the front firewall to the back of the front floor pan behind the seat.

- Must have a firewall between the trunk area, and the driver’s compartment.

- All holes must be sealed (firewall, floor pans, and rear firewall etc..). Trunk floor and rear wheel wells can be removed.

- No lightening or shortening of frame must have full frame. Can replace rear frame rails with square tubing behind rear-end.

- Quick steer and aftermarket steering are allowed.

WHEELS TIRES & BRAKES

- 8 in. maximum racing wheels. Bead locks are allowed. No homemade wheels allowed. 1 in. lug nuts on all 4 wheels.

- All 4 wheels must be the same size. No staggering of wheels.

- DOT tires or 8in Hoosiers or American racers.

- Must have all 4 working brakes. Brake Bias allowed but cannot be adjusted by driver.

ENGINE & TRANSMISSION

- The engine must be in stock location.

- V8 engines must be the same make of car ex. (GM-GM, FORD-FORD).

- Chevy’s must run a 305 only. Ford’s must run a 302 only. Olds must run 307 only.

- Unaltered quadrajet or a Holley 4412 2-barrel carburetor or a Dodge 318 only. Carbs will be gauged.

- 1 in. carburetor spacer/adapter allowed if you run the 4412.

- Stock cast iron dual plain intake. No aluminum, Highrise, boat or bowtie intakes allowed.

- No porting or polishing of the intake.

- Any HEI distributor is allowed. No MSD boxes, and no ignition boxes. No external Coils. Soft Tech Revlimiter Allowed.

- Stock factory cast iron heads. No porting or polishing. No vortec heads. No alter of Combustion of chamber for valve clearance in head.

- 1.94 and 1.5 valves only. Stock 1.5 rocker arms must have stock markings. May run screw in or pin in studs

- No aftermarket engine blocks.

- Stock cast iron manifolds. No headers or aftermarket manifolds allowed.

- Camshaft must be hydraulic only with a maximum of a .410 lift. Nowhere on the cam rotation should it exceed more than 410 lift.

- Must have stock length push rods (no shortening or lengthening of push rods).

- Flat top or dish pistons only with a maximum bore of 0.60.

- 5.7 I beam connecting rods.

- Absolutely no stroked or destroked engines. Pistons cannot come out of the block.

- Stock stroke stock appearing no lightweight crankshaft minimum of 48lbs.

- You can run racing oil pan, valve covers, air filters, aluminum pulleys, and aluminum radiators.

- No other high-performance racing parts.

- Stock transmissions with stock working torch converter. Stock manual transmissions are allowed with stock flywheel, pressure plate, and clutch.

- No racing clutches or lightened flywheels.

REAR-ENDS & SUSPENSION

- Stock rear ends or may run 9 in. ford floater. May run locked rear ends.

- Rear end, mounts, and suspension must be in the stock location.

- May run any suspension spring provided it is in the stock location.

- No wedge bolts on front or rear. No spring spacers or spring buckets allowed.

- Stock trailing arms can be boxed and can run stock length aftermarket trailing arms. No adjustable arms or offset bushings. Mounts and trailing arms must be in the stock location. 2 ¾ inch maximum from bottom of rear end housing to center of control arm bolt.

- Stock upper and lower A arms.

- May run nonadjustable tubular upper A arms with unaltered mounts but, must be stock length.

- No adjustable or rebuildable shocks. No straighter valves or bulb shocks, and no Heim joint shocks. $75 per shock non deniable claim. Track has the right to claim shocks during any racing event. 1 shock per wheel in stock location.

If you are disqualified for the following Motor, Transmission, suspension, or the refusal of a tear down or any claim you will lose all points and any winnings for the night.