Bomber Rules

No high - Performance parts  allowed unless stated otherwise!

CHASSIS

Must remain stock for the year and model of the car and all mounted in OEM hardness rubber bushings. Wheel base minimum 107'' Tolerance 1/2 inch side to side no front wheel drive.

SUSPENSION

All suspension mounts must be stock. No altering front upper mounts! Front uppers must be stock for make an model along with the front lowers. Rear Arms may be square speedway style stock length with rubber bushings ONLY! Rear arm can be no lower than 2 3/4 inch from bottom of rear housing. Again ALL suspension mounts Must be stock No wedge bolts!!

SPRINGS

Racing springs are allowed must be double pig tail on the rear. Not exceed stock length for make and model of vehicle. No Chrysler leaf springs and no composite leaf springs no adjustable lowering blocks. Non adjustable spacers allowed. No jack bolts or adjustable cups front or rear. Rear spring buckets must be flush with frame.

SHOCKS

No SMX bilsteins shocks or no other racing shocks. No Racing Shocks!!! Shocks must be in stock location with one mounting point at each end rubber only. claim rule $80 each. Shocks must be mounted even across the rear end if using a ford rear end!! 4 1/2 inches below axel tube.

REAR AXLE

May run locked or Posi Rear Ends.

Any factory type rear axle may run 9'' Floater rear axle. May use rear Disc Brake calipers must be factory type. Rotors and hats must be steel.

FRAME

Behind rear axle may be replaced with square tubing due to rust or excessive damage sub frame connectors allowed on unibody cars.

ROLL CAGE

4 Points Roll Cage required minimum 1.5 inch OD tubing with min 4 bars on driver side and 3 on passenger side no offset side to side or front to rear.

BODY

Stock Steel Body ONLY!! In stock location, Steel sheeting may be used for repair, but must be stock appearing, must have stock roof and rear pillars and trunk Lid.  5'' Max spoiler. NO EXCEPTIONS. Full stock firewall, Floor Pan must run behind driver's seat. Decking allowed to rear of seat. After market bumpers okay. Nose must match car Monte nose to Monte or caprice. Camaro to camaro. No open Backs.

BRAKES

Aftermarket brake pedal,  single Master cylinder allowed must have 4 wheel brakes no brake bias adjusters or cut offs.

ENGINE

Must be in stock Location. Stock Size Cubic inch for make and model of car. Bore not to exceed  0.60.

Stock stroke for engine claimed, all casting numbers must be visible no lightweight parts. Must have stock block.

*Crankshaft

must be stock. 1 Piece rear main allowed. No lightening may be balanced no stroker cranks!!

*Rods

Must be stock or stock replacement 5.7 press pin. No Bushed Rods

*Pistons

4 Valve flat top or dished stock replacement. No metric piston rings. No float wrist pins.

*Camshaft

Hydraulic flat tappet 450 Lift max measured at the push rod, Stock ratio, stock rocker arms only, no roller or roller tips. Must run stock length push rods.

*Intake

Stock cast iron 2 or 4 BBL, No porting or polishing. Adapter, or spacer. Max Spacer 1 inch.

*Heads

open chamber only no Vortec, double hump, or close chamber heads. 70CC min chamber Steel valves and retainers only no porting polishing grinding etc. Three angle valve job permitted stock diameter valve springs. Max Valve size for any engine 1.94 intake 1.50 exhaust. Cranking Compression 160 MAX anytime. Screw in studs allowed. No guide plates.

*Exhaust

No Headers stock exhaust manifolds ONLY!

IGNITION

Stock no MSD type boxes or MAG. Hei only! No external coils.

Carburetor

4412 Must be Gauge Legal (Will be Checked) or 750 Quadrajet max. Must have choke horn. No removable air bleeds. No billet parts. Any car owner of a car in the feature race may claim the carburetor of any car finishing ahead of the claimers car, in the feature on lead lap of feature.

Race for $500 and swap of claimers carburetor. (Tech inspector will have final say so if carburetor is in shape to swap) Any oil pan valve cover and air cleaner.

TRANSMISSION

Auto or standard

*Auto must be produced buy manufacturer of vehicle must have all working gears and torque converter 11 inch min. 38LBS. Min Diameter and Weight

*Standard must have all working gears produced by manufacturer of vehicle. Stock style steel flywheel only. 10.5 inch clutch min. Flywheel and clutch 28LBS min.

Standard Trans must add 100LBS to weight of car. Can use hydraulic slave cylinder.

TIRES

H500, M60's, or Radial Tires max size 275/60/15 radial tires receive 75LB weight break must have all 4 matching. No tire softening at all!! Grooving and sipping allowed.

WHEELS

8 Inch Steel Wheels Max bead locks on right side ONLY

FUEL

Gasoline ONLY, 110 Max, No E-85

WEIGHT

3300LBS with driver any time any vehicle not meeting the rules may have a weight added by the track official.

Anything Not covered must have approval by track officials and will be declared legal or not legal by the officials.

PROTEST/CLAIM RULES

*Shock claim/swap $80 per shock (must claim all four)

*Carburetor claim/swap $500 and swap with other driver. Or $50 Protest ONLY (Track keeps $25) The winners carb can be claimed an the claimer must finish in top 3 on lead lap and there carb must be legal! One carb claim per year per car! Tech will have say if carbs are in shape to swap!

*Engine claim/Swap $2000! This will be the long block only! no bolt on accessories beside the harmonic balancer! Claiming cars engine must finish the race with good oil pressure an temp. Any car can claim any engine but must be on lead lap. The cars must run 3 races prior to claim or being claimed.

*Engine protest (Track keeps $50) Check cam lift $100 Check Cubic Inch (Pump Engine) $150. Pull Head check stroke & Bore $250 Pull engine check rods pistons $550.